Managing stored wheat

Source: Jim Herbek, extension grain crops specialist

Unless producers take their wheat crop directly to market, the work is not finished when harvest is complete. Stored wheat needs proper care to maintain quality. Sanitation, aeration and monitoring are crucial points to remember when storing your grain during the summer months.

Totally remove the old crop before placing newly harvested wheat into a bin. Thoroughly sweep the bin wall and floor (including under aeration ducts, if possible) to remove grain kernels that may contain insect larvae or mold spores.  A wet-dry vacuum cleaner helps ease the drudgery of this chore a bit. Apply an approved insecticide both inside and outside the bin to delay insect population development before placing wheat in the bin.

Clean all equipment used to handle grain - combines, carts, trucks and receiving pits/hoppers - thoroughly to remove old grain, trash and debris that might contaminate the new crop. Use pressurized air/water. Check for holes and cracks in bin roofs and walls. Seal them to prevent leaks and entry of insects and rodents.

Grain should be cleaned and stored at 12.5 percent moisture if it will be held into August. Consider applying a grain “protectant" to the bulk grain mass. Only apply to unheated wheat. Consider applying a "Cap Out" treatment to the grain surface (Do not apply if a protectant is used). Information on currently approved products is available at http://www.uky.edu/Agriculture/Entomology/entfacts/fldcrops/ef145.htm

Aeration should be used to cool wheat after drying with heated air. To a small degree, aeration will control grain temperature if it starts heating during storage, but this may only be a short-term solution. If heating cannot be controlled by running the fan, the crop must be moved to another bin (if possible) to break up hot spots in the bin that usually cause the problem.

Check the condition of stored wheat once a week during hot weather to guard against deterioration from molds or insects. Run the fan for a few minutes to check for off odors of the air from the grain pile. Lock out unloading auger motor switches before looking inside any bin to check for wet spots on the grain surface.

Feel the top 6 to 12 inches of wheat to monitor temperatures and insect and mold activity. Use "pitfall” traps to monitor insect activity. Use three traps per bin. Check traps weekly in July and August. In colder months, trap for four days each month.

If insects are detected, have them identified and classified as primary or secondary feeders. Be especially interested in lesser grain borer or true "weevils." If insects are numerous enough to result in a discount or you are planning to hold the gain into the next warm season, consider having the grain fumigated. Always wear dust protection masks when cleaning bins and during inspections. See University of Kentucky extension publications “Principles of Grain Storage” (AEN-20) and “Aeration, Inspection, and Sampling of Grain in Storage Bins” (AEN-45) for more information on grain storage and safety considerations when inspecting stored grain.

For more information on storing grain, contact the (YOUR COUNTY) Cooperative Extension Service.

Educational programs of the University of Kentucky Cooperative Extension Service serve all people regardless of race, color, age, sex, religion, disability or national origin.

-30-